# College Network Firewall & DHCP Configuration – Support Document

## 1. Objective

This document explains the security modifications and DHCP configuration cleanup done on the Cisco 2911 Router to strengthen the college campus network. It ensures: - Proper VLAN segmentation. - Firewall-like security using ACLs. - Correct DHCP pools (no conflicts). - Support for a DMZ (Demilitarized Zone).

## 2. Key Issues Identified

- Duplicate DHCP Pools

  - Multiple pools were defined for the same networks (e.g., admin-pool and ADMIN-POOL).

  - This created redundancy and possible conflicts.

- VLAN–DHCP Mismatch

  - Some VLANs (e.g., VLAN 30 and VLAN 50) were given different pool names (ab1-pool, labs-pool) that overlapped with newer pools.

- Missing DMZ DHCP Pool

  - VLAN 70 (DMZ) had no DHCP pool, so servers could not receive automatic IPs if required.

- Unrestricted DMZ Traffic

  - Students/Wi-Fi were restricted using ACLs, but DMZ could still initiate connections into student/Admin VLANs.

## 3. Corrected DHCP Configuration

Each VLAN now has exactly one DHCP pool, avoiding overlaps. Old/lowercase pools should be removed using:

no ip dhcp pool admin-pool
no ip dhcp pool it-pool
no ip dhcp pool ab1-pool
no ip dhcp pool library-pool
no ip dhcp pool labs-pool
no ip dhcp pool canteen-pool

### Final DHCP Pools

ip dhcp excluded-address 192.168.1.1
ip dhcp excluded-address 192.168.2.1
ip dhcp excluded-address 192.168.3.1
ip dhcp excluded-address 192.168.4.1
ip dhcp excluded-address 192.168.5.1
ip dhcp excluded-address 192.168.6.1
ip dhcp excluded-address 192.168.70.1

ip dhcp pool ADMIN-POOL
 network 192.168.1.0 255.255.255.0
 default-router 192.168.1.1
 dns-server 8.8.8.8

ip dhcp pool IT-POOL
 network 192.168.2.0 255.255.255.0
 default-router 192.168.2.1
 dns-server 8.8.8.8

ip dhcp pool STUDENTS1-POOL
 network 192.168.3.0 255.255.255.0
 default-router 192.168.3.1
 dns-server 8.8.8.8

ip dhcp pool LIBRARY-POOL
 network 192.168.4.0 255.255.255.0
 default-router 192.168.4.1
 dns-server 8.8.8.8

ip dhcp pool STUDENTS2-POOL
 network 192.168.5.0 255.255.255.0
 default-router 192.168.5.1
 dns-server 8.8.8.8

ip dhcp pool WIFI-POOL
 network 192.168.6.0 255.255.255.0
 default-router 192.168.6.1
 dns-server 8.8.8.8

ip dhcp pool TEACHERS-POOL
 network 192.168.20.0 255.255.255.0
 default-router 192.168.20.1
 dns-server 8.8.8.8

ip dhcp pool DMZ-POOL
 network 192.168.70.0 255.255.255.0
 default-router 192.168.70.1
 dns-server 8.8.8.8

## 4. ACL Firewall Implementation

Since Zone-Based Firewall is not available in Packet Tracer’s 2911 IOS, extended ACLs are used.

### Example – Students VLAN (30)

ip access-list extended STUDENTS-FILTER
 permit tcp 192.168.3.0 0.0.0.255 192.168.70.0 0.0.0.255 eq 80
 permit tcp 192.168.3.0 0.0.0.255 192.168.70.0 0.0.0.255 eq 443
 permit udp 192.168.3.0 0.0.0.255 192.168.70.0 0.0.0.255 eq 53
 deny ip 192.168.3.0 0.0.0.255 192.168.1.0 0.0.0.255
 deny ip 192.168.3.0 0.0.0.255 192.168.2.0 0.0.0.255
 permit ip 192.168.3.0 0.0.0.255 any

### Example – Wi-Fi VLAN (60)

ip access-list extended WIFI-FILTER
 permit tcp 192.168.6.0 0.0.0.255 192.168.70.0 0.0.0.255 eq 80
 permit tcp 192.168.6.0 0.0.0.255 192.168.70.0 0.0.0.255 eq 443
 permit udp 192.168.6.0 0.0.0.255 192.168.70.0 0.0.0.255 eq 53
 deny ip 192.168.6.0 0.0.0.255 192.168.1.0 0.0.0.255
 deny ip 192.168.6.0 0.0.0.255 192.168.2.0 0.0.0.255
 permit ip 192.168.6.0 0.0.0.255 any

### DMZ Protection

ip access-list extended DMZ-FILTER
 deny ip 192.168.70.0 0.0.0.255 192.168.1.0 0.0.0.255
 deny ip 192.168.70.0 0.0.0.255 192.168.2.0 0.0.0.255
 deny ip 192.168.70.0 0.0.0.255 192.168.3.0 0.0.0.255
 deny ip 192.168.70.0 0.0.0.255 192.168.5.0 0.0.0.255
 deny ip 192.168.70.0 0.0.0.255 192.168.6.0 0.0.0.255
 permit ip 192.168.70.0 0.0.0.255 any

Apply outbound on VLAN 70:

interface g0/0.70
 ip access-group DMZ-FILTER out

## 5. Security Benefits

- DHCP conflicts removed – each VLAN has one clear DHCP pool.

- Students/Wi-Fi restricted – can only access DMZ for web & DNS.

- Admin & IT protected – not reachable from Students or Wi-Fi.

- DMZ isolated – cannot initiate attacks into internal VLANs.

- Future-ready – NAT & internet access can be added easily.

## 6. Next Steps

- Test DHCP allocation in each VLAN (PCs should get correct subnet addresses).

- Test ACLs: Students should reach DMZ web, but not Admin/IT.

- Secure router access (set SSH + strong passwords).

- Apply switch-side protections (port-security, DHCP snooping, STP safeguards).

Prepared for: College Campus Network Project
By: Network Support Documentation Team